import os

import pandas as pd

import numpy as np

import matplotlib.pyplot as plt

from matplotlib.ticker import MultipleLocator

from scipy.signal import butter, lfilter, detrend, iirnotch

# -------------------- Lowpass Filter 정의 --------------------

def butter_lowpass(cutoff, fs, order=5):

nyq = 0.5 * fs

normal_cutoff = cutoff / nyq

b, a = butter(order, normal_cutoff, btype='low', analog=False)

return b, a

def butter_lowpass_filter(data, cutoff, fs, order=5):

b, a = butter_lowpass(cutoff, fs, order=order)

data = pd.to_numeric(pd.Series(data), errors='coerce')

data = data.ffill().bfill().to_numpy(dtype=float)

y = lfilter(b, a, data)

return y

# -------------------- Notch Filter (근전도 고주파 억제 / 60Hz 제거) --------------------

def notch_filter(data, fs, freq=60.0, q=30.0):

"""

freq: 잡음 주파수(한국 전력 60Hz)

q: 품질계수(Q-factor) → 클수록 좁게 필터링

"""

b, a = iirnotch(freq / (fs / 2), q)

return lfilter(b, a, data)

# -------------------- 드리프트 제거 --------------------

def remove_drift(data, method="detrend", window_sec=1, fs=250):

data = np.array(data, dtype=float)

if method == "detrend":

return detrend(data)

elif method == "moving":

window = max(int(window_sec * fs), 1)

baseline = pd.Series(data).rolling(window, center=True, min_periods=1).mean()

return data - baseline.to_numpy()

else:

return data

# -------------------- 데이터 로드 --------------------

base_dir = r"C:\Users\admin\Documents\OpenBCI_GUI\Recordings"

folder_paths = [f.path for f in os.scandir(base_dir) if f.is_dir() and not f.name.startswith('.')]

def load_data(folder_path):

df_list, filename_list = [], []

for file_name in os.listdir(folder_path):

if file_name.endswith('.csv'):

filename_list.append(file_name)

file_path = os.path.join(folder_path, file_name)

df_list.append(pd.read_csv(file_path, sep="\t", header=None))

return df_list, filename_list

print("=== Protocol Folders ===")

for i, path in enumerate(folder_paths):

print(f"{i}: {os.path.basename(path)}")

ex_idx = int(input(f"Choose a protocol (0-{len(folder_paths)-1}): "))

folder_path = folder_paths[ex_idx]

df_list, filename_list = load_data(folder_path)

print('=== Experiments List ===')

for i, ex in enumerate(filename_list):

if i % 3 == 0: print("\n")

print('%3d|%s' % (i, ex), end="")

exp_no = int(input('Choose an experiment number (0-%d): ' % (len(df_list)-1)))

df_bf = df_list[exp_no]

# -------------------- BrainFlow 데이터프레임 구성 --------------------

exg = df_bf.iloc[:, 1:9]

ts_bf = df_bf.iloc[:, -2].astype(float)

bf_data = pd.concat([ts_bf, exg], axis=1)

bf_data.columns = ["timestamp"] + [f"ch{i}" for i in range(8)]

# -------------------- 설정 --------------------

fs = 250

n_samples = len(bf_data)

t_rel = np.arange(n_samples) / fs

cutoff = 10

order = 5

use_notch = True   # 60Hz 필터링 켜기/끄기 옵션

# -------------------- Plot --------------------

for ch in range(8):

raw = bf_data[f"ch{ch}"].to_numpy()

# Notch filter 적용 (근육 포함 고주파 억제)

if use_notch:

raw = notch_filter(raw, fs=fs, freq=60.0)

# Lowpass 필터 적용 (근육 신호 제거)

y_filt = butter_lowpass_filter(raw, cutoff=cutoff, fs=fs, order=order)

# 드리프트 제거

y_clean = remove_drift(y_filt, method="detrend")

# Plot

plt.figure(figsize=(12, 4))

plt.plot(t_rel[t_rel >= 5], y_clean[t_rel >= 5], linewidth=1.4, label=f"ch{ch + 1}")

plt.title(f"BrainFlow EXG Channel {ch + 1} (Muscle Removed + Drift Removed)")

plt.xlabel("Time (s)")

plt.ylabel("Amplitude (µV)")

plt.grid(True)

ax = plt.gca()

ax.xaxis.set_major_locator(MultipleLocator(5.0))

ax.yaxis.set_major_locator(MultipleLocator(1500))

plt.legend(loc="upper right")

plt.tight_layout()

plt.show()